FINANCIAL REQUIREMENTS

A breakdown of the main cost items in the local currency for drilling a bore hole. (One borehole)

| ITEM | DESCRIPTION | UNIT | QTY | RATE KSH | AMOUNT (KSH) |
| --- | --- | --- | --- | --- | --- |
| A | Mobilization |  |  |  |  |
| A1 | Allow for confirmation survey | Item | 1 | 45,000 | 45,000 |
| A2 | Mobilization and demobilization of all plant, materials, equipments and personnel | Item | 1 | 308,000 | 308,000 |
| A3 | Setting up and dismantling | No. | 1 | 48,000 | 48,000 |
| B | Drilling |  |  |  |  |
| B1 | Drilling at 254mm diameter | M | 5 | 6,500 | 32,500 |
| B2 | Drilling at 203mm diameter | M | 75 | 3,900 | 292,500 |
| B3 | Reaming (all diameters, when instructed) | M | - | 114,000 | 114,000 |
| B4 | Sampling and storing of drill cutting at 2m interval | M | 40 | 300 | 12,000 |
| C | Supply of materials |  |  |  |  |
| C1 | Supply of surface casing, mild steel 5mm thickness, 209mm ID, 5m lengths | M | 5 | 4,000 | 20,000 |
| C2 | Supply of plain casing ,UPVC, 160mm OD | LM | 60 | 5,000 | 300,000 |
| C3 | Supply of perforated screens, UPVC 160MM, OD | LM | 20 | 3,850 | 77,000 |
| C4 | Supply of centralizers , 203/160mm | N o | 5 | 6,000 | 30,000 |
| C5 | Supply of bottom caps | No | 1 | 6,000 | 6,000 |
| C6 | Supply of Gravel pack | M3 | 8 | 8,000 | 64,000 |
| C7 | Supply of inert backfill | M3 | - | 18,000 | 18,000 |
| C8 | Supply of Cray seals | M3 | - | 120,000 | 120,000 |
| D | Installation |  |  |  |  |
| D1 | Installation of surface casing and screens, mild steel 209m 1d | M | 5 | 820 | 4,100 |
| D2 | Installation of casing and screens, UPVC 160MM OD, including centralizers and bottom cups | M | 80 | 1,000 | 80,000 |
| D3 | Installation of Gravel backfill | M3 | 10 | 800 | 80,000 |
| D4 | Installation of Gravel pack | Sum | - | 43,000 | 43,000 |
| E | Development and Testing Pumping |  |  |  |  |
| E1 | Well development and airlifting (6hrs) | Hr | 6 | 7,600 | 45,600 |
| E4 | Constant discharge test 24hrs | Hr | 24 | 3,800 | 91,000 |
| E5 | Recovery 12hrs | Hr | 12 | 1,000 | 12,000 |
| F | Well Completion |  |  |  |  |
| F1 | Well disinfections, Quality analysis report and Borehole completion report | Item | Sum | 15,000 | 15,000 |
| F2 | Borehole cover | No. | 1 | 10,000 | 10,000 |
| F3 | Slab 1m× 1m × 2ft | No. | 1 | 15,800 | 15,800 |
|  | TOTAL |  |  |  | 1,883,500 |
|  | 1 Solar pump, pipes and fitting |  |  |  | 688,000 |
|  | GRAND TOTAL |  |  |  | 2,571,500 |